SAMPLE VFR

- REQUIREMENTS

Files:

The Sample VFR Digital Files are 2400 DPI, CCITT T.6 compressed, prescreened, 1bit TIFF with file dimensions of 59.786 (width) x 45.000 (height) inches.  Ten files furnished.  A  non-step-up and step-up TIFF file required for each color per chart.

Platemaking:

For the Sample VFR, the plate size should be a minimum of 60 (width) x 47 (height) inches to accommodate digital file size. Plating process will require the merging of the non-step-up and step-up digital files on each plate and the shifting of the step-up digital file ½ inch on the Y axis of the plate.  See pdf attachment with “Sample instructions” for clarification of step-up look of center trim area.

Printing:

The Sample VFR chart will print as “Work-and-Tumble” process and is formatted “one up, two out per press sheet.  For “Sample Only” the following PMS colors can be used to closely match inks.    Black: Pantone, Brown: PMS 1385U, Blue: PMS 307U, Magenta: PMS 259U, Yellow: Process.  NOTE: Formulated inks will be used when bid is awarded.  The press sheets may differ from the SOW requirements due to time restraints.  Effort to achieve close equivalent to paper poundage (lbs) should be made.

Finishing:

Trimming and folding Sample VFR see Section 4.5 Task 5: Finishing in the SOW.